climada storm surge from tropical cyclones

Dr. David N. Bresch, Head Sustainability, Swiss Re

Lea Müller, Climate Change Specialist, Swiss Re

11 June 2013

Content

1.1.	TC storm surge literature	1

1.2.	SLOSH	1

1.3.	Important Factors	2

1.4.	Nomograms	4

1.5.	Elevation data	6

### TC storm surge literature

Slosh homepage (Nomograms vom Jelesnianski)

NOAA

http://www.ngdc.noaa.gov/mgg/coastal/coastal.html

http://www.csc.noaa.gov/digitalcoast/data/search/elevation/high-resolution

http://www.csc.noaa.gov/digitalcoast/dataregistry/#/shallowwaterbathy

http://maps.ngdc.noaa.gov/viewers/wcs-client/

http://www.ngdc.noaa.gov/mgg/coastal/grddas03/grddas03.htm

THE APPLICATION OF SPLASH MODEL NOMOGRAM METHOD TO THE COAST OF CHINA

http://slosh.nws.noaa.gov/sloshPub/

http://en.cnki.com.cn/Article_en/CJFDTOTAL-HUTB198304004.htm

The Influence of Storm Size on Hurricane Surge

http://journals.ametsoc.org/doi/pdf/10.1175/2008JPO3727.1

On the importance of the forward speed of hurricanes in storm surge forecasting: a numerical study

http://onlinelibrary.wiley.com/doi/10.1029/2008GL036953/abstract;jsessionid=BD9468BD70717722AE80B0194C9A4644.d04t04?deniedAccessCustomisedMessage=&userIsAuthenticated=false

PROBABILISTIC GUIDANCE FOR HURRICANE STORM SURGE

http://slosh.nws.noaa.gov/sloshPub/pubs/psurge_ofcl_200801_AMS.pdf

Data for validation

Peaksuge database "surgedat"

### SLOSH

An operational forecast model for extraropical storm surges along the US: East Coast

http://slosh.nws.noaa.gov/sloshPub/pubs/etsurge_1996OpnlForecast.pdf

SLOSH – a hurricane storm surge forecast model

http://ieeexplore.ieee.org/xpl/login.jsp?tp=&arnumber=1152341&url=http%3A%2F%2Fieeexplore.ieee.org%2Fxpls%2Fabs_all.jsp%3Farnumber%3D1152341

http://www.nhc.noaa.gov/surge/slosh.php#INTRO

hurricanes by taking into account the atmospheric pressure, size, forward speed, and track data. These parameters are used to create a model of the wind field which drives the storm surge.

http://www.nhc.noaa.gov/surge/

Jelesnianski 1992

http://slosh.nws.noaa.gov/sloshPub/pubs/SLOSH_TR48.pdf

### Important Factors

What impacts storm surge and how are these impacts generalized?

Intensity (higher winds speeds=increased storm surge)

Central pressure (little impact)

Forward speed (slower storms=higher and broader storm surge inland including bays and estuaries; faster storms=more storm surge along the open coast)

Size (storm with large wind field=more storm surge; storm with small wind field=less storm surge)

Angle of approach (perpendicular to coastline=more storm surge; parallel to coastline=less storm surge)

Width and slope of continental shelf (wide shelf/gentle slope=more storm surge with relatively small waves; narrow shelf/sharp slope=less storm surge with relatively big waves)

- Topography of ocean bed, depth of the sea  Digital elevation model (DEM), transform steep versus shallow sea bed into a factor

SRTM 90m Digital Elevation Data

http://srtm.csi.cgiar.org/

Resampled SRTM data to 250m resolutions for the entire globe are available (Password: ThanksCSI!)

https://hc.box.net/shared/1yidaheouv

https://hc.box.com/shared/1yidaheouv#/s/1yidaheouv/1/34252608/353949440/1

http://srtm.jrc.ec.europa.eu/

http://nationalmap.gov/viewer.html

Local features (concavity of coastlines, bays, rivers, headlands, islands, etc.=greater storm surge impact); Type of coastline: concave versus convex

Figure 1. Source http://slosh.nws.noaa.gov/sloshPub/SLOSH-Display-Training.pdf#page=14

Several generalizations can be drawn from SLOSH models.

More intense storms cause higher surges.

The highest surges usually occur to the right of the storm track (traveling with the storm) at approximately the radius of maximum wind.

Fast moving storms cause high surges along open coast and lower surges in sheltered bays and estuaries.

Slow moving storms usually result in greater flooding inside bays and estuaries, with smaller values along the open coast.

Larger storms affect longer stretches of the coastline. A larger storm is defined as a storm having a greater radius of maximum wind.

The direction of storm approach often impacts the extent of flooding. Depending on location, storms from one direction may cause inundation, while a storm of the same magnitude from a different direction may cause little flooding.

The slope of the continental shelf will affect the level of surge in a particular area. Areas with shallow slopes of the continental shelf will allow a greater storm surge, but waves are small.

Areas with deep water just offshore experience large waves, but little storm surge.

### Nomograms

Global Guide to Tropical Cyclone Forecasting:

CHAPTER 4: THE HABITATION LAYER

4.2 STORM SURGES

http://cawcr.gov.au/bmrc/pubs/tcguide/ch4/ch4_2.htm

Figure 2: Nomogram for peak surge on the open coast. Entering arguments are pressure drop and radius of maximum winds. The maximum winds are valid for 10-min average at 10 m elevation for a stationary cyclone over water. The curves were computed for a standard tropical cyclone motion across a standard basin, as described in the main text. Source: http://cawcr.gov.au/bmrc/pubs/tcguide/ch4/ch4_figures/figure4_3.htm

Figure 3: Nomogram of correction factors against vector tropical cyclone motion. The factor corrects Fig. 4.3 for non-standard tropical cyclone motion and the inset orientates the cyclone track angle relative to a coast. Source: http://cawcr.gov.au/bmrc/pubs/tcguide/ch4/ch4_figures/figure4_4.htm

Figure 4: Nomogram of shoaling correction factors for the Gulf coast of the USA. The factor corrects Fig. 4.3 for real, non-standard basins. Source: http://cawcr.gov.au/bmrc/pubs/tcguide/ch4/ch4_figures/figure4_5.htm

### Elevation data

Figure 5: Elevation model of Florida. Data source: NOAA, US Coastal Relief Model, Florida and Eastern Gulf of Mexico, http://www.ngdc.noaa.gov/mgg/coastal/grddas03/grddas03.htm, http://www.ngdc.noaa.gov/mgg/coastal/crm/data/arc_ascii/fl_east_gom_crm_v1.asc.zip . The region was selected through the map interface http://ngdc.noaa.gov/dem/squareCellGrid/map#

Figure 6: Elevation model of Puerto Rice. Data source: NOAA, 2004 USACE: Puerto Rico. http://www.csc.noaa.gov/digitalcoast/dataregistry/#/, http://maps.ngdc.noaa.gov/viewers/wcs-client/